ELECTROANALYTICAL METHOD coupled with chemometry FOR DETECTION OF virgin olive oil adulteration

# Constantin Apetrei1, Elisabeta-Irina Geană2, Irina Mirela Apetrei3

## 1- Faculty of Sceinces and Environment, Dunarea de Jos University of Galati, 47 Domneasca, Galati, Romania.

## 2- National Research and Development Institute for Cryogenics and Isotopic Technologies - ICSI Rm. Valcea, Uzinei Street no. 4, Ramnicu Valcea, Romania.

## 3- Faculty of Medicine and Pharmacy, Dunarea de Jos University of Galati, 47 Domneasca, Galati, Romania.

## apetreic@ugal.ro

Background:  The detection of olive oil adulteration with lower quality oils continues to be an important problem in the olive industry since fraud continues to be committed. The current methods used to detect these adulterations require complicated previous steps and are time and money consuming. Furthermore, these needs high qualified persons. Therefore, there is a serious need to develop simpler, faster and cheaper methods for this purpose.

Aim: In the present research the potential of a novel electrochemical method based on screen printed electrodes modified with nanomaterials for the detection of extra virgin olive oil (EVOO) adulteration was developed.

Method: The screen printed electrodes were modified with oils under study and the cyclic voltammograms were recorded in two different electrolyte solutions. The data were processed by means of multivariate data analysis and classification models were developed.

Results and discussion: Extra virgin olive oil and samples containing different ratios of other oils such as pomace oil, sunflower, high oleic sunflower and corn were analysed. The electrochemical signals of the sensors immersed in the electrolyte solution are chemical fingerprints of the oils under study. These are mainly related with the presence of phenolic compounds and other compounds with antioxidant character from the samples. The PLS-DA and SIMCA models developed have shown that the samples could be correct classified in agreement with the EVOO contents, with an error below 2%. Probabilistic Neural Networks models success rate (correct predicted over total number of samples) was 99% for all adulterate oils analysed.

Conclusion: The electroanalytical method coupled with chemometrics could be used for the accurate detection of olive oil adulteration.

Acknowledgments. This work was supported by a grant of the Romanian Ministry of Education and Research,

CNCS—UEFISCDI, project number PN‐III‐P4‐ID‐PCE‐2020‐0923, within PNCDI III.

Keywords: extra virgin olive oil, sensor, data analysis.